Greenhouse State Monitoring API – MQTT Specification

Version: 1.0
Purpose: Secure IoT communication between a single Raspberry Pi–based greenhouse controller and a cloud backend (FastAPI).

0. Broker & Connection Details

MQTT Broker

- Host: <YOUR_VM_PUBLIC_IP or DNS>

- Port: 8883

- Protocol: MQTT over TLS (MQTTS)

- Broker software: Mosquitto

- Plain MQTT (1883): ❌ Disabled

Authentication

- Username: greenhouse-mqtt

- Password: greenhouseiot2026 (practice only)

TLS

- TLS version: TLS 1.2 or newer

- Server authentication: Required

- CA certificate: Provided by broker (self-signed or trusted CA)

Client must validate the broker certificate.

1. Device Identity & Online Status

Even with one device, identity is still important for:

- clarity

- logs

- future expansion

- clean architecture

Device ID

device_id = "greenhouse-raspi-1"

This ID is:

- Included in payloads

- Used for online/offline tracking

- Stable across reboots

2. Connection Lifecycle (Alive / Dead)

2.1 Online (Birth Message)

Published automatically on successful connection.

- Topic:
greenhouse/state/connection

- Payload (JSON):

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:00:00Z",

"status": "ONLINE"

}

- QoS: 1

- Retain: true

2.2 Offline (Last Will / Death Message)

Configured as MQTT Last Will and Testament (LWT).

Published automatically by the broker if the device:

- loses power

- crashes

- disconnects unexpectedly

- Topic:
greenhouse/state/connection

- Payload (JSON):

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:05:00Z",

"status": "OFFLINE"

}

- QoS: 1

- Retain: true

This allows FastAPI/UI to always know if the device is alive.

3. Telemetry Topics (MCU → Broker)

3.1 Periodic Environmental Telemetry

Publish interval: every 60 seconds

- Topic:
greenhouse/state/environment

- Payload (JSON):

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:05:00Z",

"environmental_measurements": {

"ambient_light_level": 620,

"air_temperature_celsius": 24.3,

"relative_humidity_percent": 68.1

}

}

- QoS: 0 (or 1 if loss is unacceptable)

- Retain: false

4. Event Topics (MCU → Broker)

Events represent changes or actions that originate locally on the device.

4.1 Local Alarm Acknowledge (Physical Pushbutton)

- Topic:
greenhouse/events/localcontrol/acknowledge_fault

- Payload (JSON):

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:08:00Z",

"acknowledged": true

}

- QoS: 1

- Retain: false

4.2 Setpoint Change Events

4.2.1 Temperature Setpoint Change

- Topic:
greenhouse/events/setpoint/target_air_temperature_celsius

- Payload (JSON):

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:06:02Z",

"value_celsius": 23.0

}

- QoS: 1

- Retain: false

4.2.2 Humidity Threshold Change

- Topic:
greenhouse/events/setpoint/maximum_relative_humidity_percent

- Payload (JSON):

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:06:10Z",

"value_percent": 70

}

- QoS: 1

- Retain: false

4.3 Actuator Events

4.3.1 Ventilation Fan

- Topic:
greenhouse/events/actuator/ventilation_fan_enabled

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:06:20Z",

"status": true

}

4.3.2 Lamp Brightness

- Topic:
greenhouse/events/actuator/lamp_brightness_percent

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:06:25Z",

"value_percent": 80

}

4.3.3 Heating Element

- Topic:
greenhouse/events/actuator/heating_element_enabled

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:06:30Z",

"status": true

}

- QoS: 1

- Retain: false

5. Alarm Events

- Topic:
greenhouse/events/alarm

- Allowed severity levels:
green | orange | red

Normal

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:06:40Z",

"alarm_active": false,

"alarm_severity_level": "green",

"alarm_code": "OK",

"alarm_message": "OK"

}

Medium Alarm Raised

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:07:10Z",

"alarm_active": true,

"alarm_severity_level": "orange",

"alarm_code": "HUMIDITY_HIGH",

"alarm_message": "Humidity exceeded maximum threshold"

}

Alarm Raised

{

"device_id": "greenhouse-raspi-1",

"timestamp_utc": "2026-01-27T14:07:10Z",

"alarm_active": true,

"alarm_severity_level": "red",

"alarm_code": "TEMPERATURE_LOW",

"alarm_message": "Temperature below frost temperature threshold"

}

- QoS: 1

- Retain: false

6. Control Topics (Broker → MCU)

Control topics represent remote commands, typically issued by FastAPI or a user interface.

The MCU must ignore any command where:

target_device_id != "greenhouse-raspi-1"

6.1 Control Mode

- Topic:
greenhouse/remotecontrol/mode

- Payload (JSON):

{

"target_device_id": "greenhouse-raspi-1",

"source_id": "web-ui-operator-1",

"timestamp_utc": "2026-01-27T14:10:00Z",

"mode": "REMOTE"

}

- QoS: 1

- Retain: true

6.2 Target Temperature (Remote)

- Topic:
greenhouse/remotecontrol/setpoints/target_air_temperature_celsius

{

"target_device_id": "greenhouse-raspi-1",

"source_id": "web-ui-operator-1",

"timestamp_utc": "2026-01-27T14:11:00Z",

"value_celsius": 23.0

}

- QoS: 1

- Retain: true

6.3 Maximum Humidity Threshold (Remote)

- Topic:
greenhouse/remotecontrol/setpoints/maximum_relative_humidity_percent

{

"target_device_id": "greenhouse-raspi-1",

"source_id": "web-ui-operator-1",

"timestamp_utc": "2026-01-27T14:12:00Z",

"value_percent": 70

}

- QoS: 1

- Retain: true

6.4 Remote Alarm Acknowledge

- Topic:
greenhouse/remotecontrol/acknowledge_fault

{

"target_device_id": "greenhouse-raspi-1",

"source_id": "web-ui-operator-1",

"timestamp_utc": "2026-01-27T14:13:00Z",

"acknowledged": true

}

- QoS: 1

- Retain: false

7. Security Summary

| Layer | Status |
| --- | --- |
| TLS encryption | ✅ |
| Username/password | ✅ |
| Plain MQTT disabled | ✅ |
| Firewall (8883 only) | ✅ |
| Last Will (offline detection) | ✅ |